INTERNSPARK INTERNSHIP

RTL Design & Functional Simulation (Verilog)

Task 1 Report

| Name | Budde Rohini |
| --- | --- |
| College | Narsimha Reddy Engineering College |
| Task | Task 1 - RTL Design & Functional Simulation |

# 1. Goal

Design an 8-bit ALU (Arithmetic Logic Unit) in Verilog and verify its functionality using a testbench with directed test cases.

# 2. RTL Design - 8-bit ALU Module

The ALU supports 4 operations: ADD, SUB, AND, OR.

module rtl_design(

input [7:0] A,

input [7:0] B,

input [2:0] opcode,

output reg [7:0] result,

output reg carry

);

always @(*) begin

carry = 0;

case(opcode)

3'b000: {carry,result} = A + B;

3'b001: {carry,result} = A - B;

3'b010: result = A & B;

3'b011: result = A | B;

3'b100: result = A ^ B;

3'b101: result = ~A;

3'b110: result = A << 1;

3'b111: result = A >> 1;

default:

begin

result = 8'b00000000;

carry  = 1'b0;

end

endcase

end

endmodule

# 3. Testbench

`timescale 1ns/1ps

module rtl_design_tb;

reg [7:0] A;

reg [7:0] B;

reg [2:0] opcode;

wire [7:0] result;

wire carry;

rtl_design DUT(

.A(A),

.B(B),

.opcode(opcode),

.result(result),

.carry(carry)

);

initial begin

A = 0;

B = 0;

opcode = 0;

#10;

A = 8'd20;

B = 8'd10;

opcode = 3'b000;

#10;

A = 8'd25;

B = 8'd5;

opcode = 3'b001;

#10;

A = 8'b10101010;

B = 8'b11001100;

opcode = 3'b010;

#10;

opcode = 3'b011;

#10;

opcode = 3'b100;

#10;

A = 8'b11110000;

opcode = 3'b101;

#10;

A = 8'b00001111;

opcode = 3'b110;

#10;

opcode = 3'b111;

#10;

$finish;

end

endmodule

Code screenshot

Source code

## Test bench code

## 4. Simulation Output

Simulated using JDoodle Online Verilog Compiler. All operations verified correct.

| Operation | A | B | Result |
| --- | --- | --- | --- |
| ADD (10+5) | 10 | 5 | 15 |
| SUB (10-5) | 10 | 5 | 5 |
| AND (10&5) | 10 | 5 | 0 |
| OR (10¦5) | 10 | 5 | 15 |

# Output Screenshot

## 5. Simulation Waveform

Waveform generated using EDA Playground (EPWave) with Icarus Verilog 12.0. Shows correct signal transitions for all 4 operations.

## 6. Coverage Summary

All 4 ALU operations tested with directed test cases (a=10, b=5):

- ADD: 10 + 5 = 15 ✓

- SUB: 10 - 5 = 5 ✓

- AND: 10 & 5 = 0 ✓

- OR: 10 | 5 = 15 ✓

Tool: EDA Playground with Icarus Verilog 12.0 + EPWave for waveform visualization.